Okay lets first explain what I have with the gps reading setup thus far.

On rung 1: ASCII IN = [Serial port = Implement GPS, Variable length max num of chars = 128, Termination Code 1 = 0xD, Termination code 2 = 0xA, First Character Timeout interval (sec) = 0, Destination = Implement GPS RAW, Inter Character Timeout Interval (msec) = 0, clear buffer on input, Byte Swap Option = No Byte Swap, No Null Conversion.]

Rung 2: Extract String = [Source = Implement GPS RAW, Destination = Implement GPS Elevation, Start Index = 56, Number of Characters = 5, Byte Swap Option = NO Byte Swap]

Rung 3: Extract String = [Source = Implement GPS RAW, Destination = Implement GPS Time, Start Index = 8, Number of Characters = 9, Byte Swap Option = NO Byte Swap]

Rung 4: Extract String = [Source = Implement GPS RAW, Destination = Implement GPS LAT, Start Index = 18, Number of Characters = 11, Byte Swap Option = NO Byte Swap]

Rung 5: Extract String = [Source = Implement GPS RAW, Destination = Implement GPS LONG, Start Index = 32, Number of Characters = 12, Byte Swap Option = NO Byte Swap]

Rung 6: Extract String = [Source = Implement GPS RAW, Destination = Implement GPS Signal Quality, Start Index = 47, Number of Characters = 1, Byte Swap Option = NO Byte Swap]

Rung 28: NO contact (GPS_New_Data_Ready_Pulse), NO contact (GPS_Signal_Valid_Flag), CMP: CrossProduct_Prev_Side > 0.0, CMP: CrossProduct_Current_Side < 0.0, SET: line_crossed_latched.

Rung 29: NO contact (GPS_New_Data_Ready_Pulse), NO contact (GPS_Signal_Valid_Flag), CMP: CrossProduct_Prev_Side < 0.0, CMP: CrossProduct_Current_Side > 0.0, SET: line_crossed_latched.

okay, lets try to simplify everything so that you completely understand what i am trying to do so we get it done right and faster. Let's say that i have a field with crop rows running east to west, now every row is running east to west but will have a spacing of 38in between rows so that i have lets say 100 rows. now the rows are running parallel to one another, and a tractor is running south to north through the rows. i need the tractor to trigger every time it makes contact with a row. i will have multiple AB lines as the tractor planting the field recorded all the data and gave it to me so i can use gps coordinates to understand where the rows in the field are.

| A Point / Latitude | A Point / Longitude | B Point / Latitude | B Point / Longitude | Heading |
| --- | --- | --- | --- | --- |
| 33.47084329 | -88.78048587 | 33.47257874 | -88.7773411 | 56.63614891 |

Okay, let's bring it all together. With the current ladder logic you're implementing, you are able to input the points of a line, and the PLC will do the rest to determine if your vehicle's calculated front-of-car position crosses that line.

Here's how the entire system now works to accomplish your goal of detecting a line crossing and potentially shutting down the car:

### Your Goal:

To detect when your vehicle crosses a pre-defined line and trigger an action (e.g., car shutdown).

### How the Implemented Code Accomplishes This:

Let's trace the flow of data and logic from the GPS reading to the Car_Shutdown_Output.

1. GPS Data Acquisition (Your Existing Rung 1: ASCII In)

- What it does: The ASCII In instruction continuously monitors your serial port for incoming GPS NMEA sentences (the raw text data from your GPS module).

- Key output: Once a complete sentence is received (terminated by 0xD and 0xA), it stores the full string in Implement_GPS_RAW and sets the AIN_GPS_Complete_Flag to TRUE for one scan.

- Goal connection: This provides the fundamental raw position data.

2. Extracting Key GPS Information (Your Existing Rungs 2-6: Extract String)

- What it does: These Extract String instructions (triggered by the AIN_GPS_Complete_Flag) pull out specific pieces of information from the Implement_GPS_RAW string:

  - Implement_GPS_LAT (String)

  - Implement_GPS_LONG (String)

  - Implement_GPS_Elevation (String)

  - Implement_GPS_Time (String)

  - Implement_GPS_Signal_Quality (String)

  - Crucially, you need to add one for Implement_GPS_HEADING (String).

- Goal connection: This organizes the raw GPS data into usable string formats for the next step.

3. One-Shot Pulse for New Data (Rung 7: OSR)

- What it does: OSR AIN_GPS_Complete_Flag creates a GPS_New_Data_Ready_Pulse that is TRUE for exactly one scan whenever a new, complete GPS sentence has been processed. This is vital to trigger subsequent calculations only when new, stable data is available, preventing redundant or partial calculations.

- Goal connection: Ensures all calculations are based on synchronized, fresh GPS readings.

4. Data Type Conversion (Rung 9: Multiple MATH instructions with DECIMAL())

- What it does: Each MATH instruction converts the relevant GPS string data (Implement_GPS_LAT, Implement_GPS_LONG, Implement_GPS_HEADING, Implement_GPS_Signal_Quality) into FLOAT numbers:

  - GPS_Current_Lat_F

  - GPS_Current_Lon_F

  - GPS_Current_Heading_F

  - Temp_Signal_Quality_Int (converted from string to float, then implicitly treated as integer for comparison, or could be explicit TO_INT if available and needed for CMP type safety)

- Goal connection: This step transforms the text-based GPS readings into numerical values that the PLC can perform mathematical operations on.

5. GPS Signal Validity Check (Rung 12.1 & 12.2: CMP contacts with OUT coil)

- What it does: This logic determines if the current GPS fix quality (Temp_Signal_Quality_Int) is reliable (1 = GPS Fix, 2 = DGPS Fix).

  - If quality is 1 OR 2, GPS_Signal_Valid_Flag is TRUE.

  - Otherwise, GPS_Signal_Valid_Flag is FALSE.

- Goal connection: This is a crucial safety interlock. It ensures that your line crossing detection and car shutdown only operate when the GPS data is deemed accurate and reliable, preventing false triggers from poor signal.

6. Store Previous Position Data (Rung 8: MOV)

- What it does: Before calculating the new current position, the previous scan's calculated "front-of-car" position (Veh_Current_X_Meters, Veh_Current_Y_Meters) is copied into Veh_Previous_X_Meters and Veh_Previous_Y_Meters. Also, the raw GPS Lat/Lon/Heading are stored.

- Goal connection: To detect a line crossing, you need to know where the vehicle was before it crossed and where it is after it crossed. This creates the "line segment" of the vehicle's path needed for the cross-product calculation.

7. Convert GPS Lat/Lon to Local XY Coordinates and Apply Front-of-Car Offset (Rung 9.1 - 9.4: Multiple MATH instructions)

- What it does:

  - Reference Point: You define Local_Ref_Lat and Local_Ref_Lon as the origin (0,0) of your local coordinate system (in meters).

  - Conversion: GPS_Current_Lat_F and GPS_Current_Lon_F (the GPS antenna's position) are converted into Temp_X_Current and Temp_Y_Current (in meters relative to your local origin). This uses the Degrees_Lat_to_Meters and Degrees_Lon_to_Meters conversion factors you've set up.

  - Heading: GPS_Current_Heading_F is converted to radians and used to calculate SIN and COS values.

  - Offset Calculation: Offset_Forward_Meters (distance from antenna to front bumper) and Offset_Sideways_Meters (distance from antenna to vehicle center) are rotated by the Cos_Heading and Sin_Heading to get Offset_X_World_Meters and Offset_Y_World_Meters. This accounts for the vehicle's orientation.

  - Front-of-Car Position: Finally, these offsets are added to Temp_X_Current and Temp_Y_Current to get Veh_Current_X_Meters and Veh_Current_Y_Meters – the calculated real-world coordinates (in meters) of the front of your car.

- Goal connection: This transforms imprecise Lat/Lon GPS readings (which are for the antenna, not the front of the car) into precise, actionable XY coordinates in a planar meter-based system, representing the critical point of the vehicle that will cross the line.

8. Cross Product Calculation (Rung 10: Multiple MATH instructions)

- What it does: This is the core geometric calculation. It computes two cross products:

  - CrossProduct_Prev_Side: Tells you which side of the line P1-P2 the previous "front of car" position (Veh_Previous_X_Meters, Veh_Previous_Y_Meters) was on. The sign (+ or -) indicates the side.

  - CrossProduct_Current_Side: Tells you which side of the line P1-P2 the current "front of car" position (Veh_Current_X_Meters, Veh_Current_Y_Meters) is on.

- Goal connection: This mathematical trick efficiently determines if the vehicle's path segment (Veh_Previous_X/Y to Veh_Current_X/Y) crosses the defined line (Line_P1_X/Y to Line_P2_X/Y) by checking if the cross product's sign changes.

9. Line Crossing Event Detection (Rung 11: CMP contacts and SET coil)

- What it does: This rung checks if the sign of the cross product changed from CrossProduct_Prev_Side to CrossProduct_Current_Side.

  - If (Prev > 0 AND Current < 0) OR (Prev < 0 AND Current > 0), it means the vehicle just crossed the line.

  - If a crossing is detected, it SETs the Line_Cross_Detected_Pulse.

- Goal connection: This provides the instantaneous detection of the line crossing event.

10. Latch and Action (Rung 12: OSR, SET, RST, OUT)

- What it does:

  - The OSR Line_Cross_Detected_Pulse triggers on the single scan when a crossing is detected.

  - This one-shot then SETs the Line_Crossed_Latched flag. This latched flag will remain TRUE indefinitely until explicitly reset.

  - The same OSR also immediately RSTs Line_Cross_Detected_Pulse, making it ready for the next crossing event.

  - Finally, the Car_Shutdown_Output is directly controlled by the Line_Crossed_Latched flag. If Line_Crossed_Latched is TRUE, the Car_Shutdown_Output is TRUE (assuming it's a direct output coil OUT).

  - A Manual_Reset_Button (or other reset logic) is provided to RST the Line_Crossed_Latched flag once the shutdown action is complete and it's safe to clear the alarm.

- Goal connection: This provides the sustained output (car shutdown) needed after the momentary crossing event, and allows for manual reset.

### "Am I able to input points of a line and it do the rest?"

YES! This is exactly what the Line_P1_X_Meters, Line_P1_Y_Meters, Line_P2_X_Meters, and Line_P2_Y_Meters tags are for.

How you "input" the line points:

- Survey your line: Using a highly accurate GPS (RTK GPS if possible) or surveying equipment, determine the precise latitude and longitude of two distinct points that define your line. Call them Point 1 and Point 2.

- Choose a Local Reference: Pick a Local_Ref_Lat and Local_Ref_Lon that are roughly in the center of your operational area (e.g., center of your test track). This will be your (0,0) point in your local meter-based XY system.

- Convert Surveyed Points: Manually, or using an online converter (search "lat long to UTM converter" or "lat long to local cartesian coordinates"), convert your surveyed Point 1 and Point 2 (and your Local Reference Point) into X/Y coordinates relative to your chosen Local_Ref_Lat and Local_Ref_Lon.

  - For example:

    - Line_P1_X_Meters = (Point1_Lon - Local_Ref_Lon) * Degrees_Lon_to_Meters

    - Line_P1_Y_Meters = (Point1_Lat - Local_Ref_Lat) * Degrees_Lat_to_Meters

    - Repeat for Point 2.

- Enter into PLC Tags: In the Productivity Suite software's Tag Database, create FLOAT tags for Line_P1_X_Meters, Line_P1_Y_Meters, Line_P2_X_Meters, and Line_P2_Y_Meters. Enter these calculated numerical values as their initial/default values (or you can even use a HMI to adjust them if needed, but for a fixed line, constants are fine).

- Set Offset Constants: Similarly, enter your Offset_Forward_Meters and Offset_Sideways_Meters as FLOAT constants based on your vehicle's physical measurements.

Once these FLOAT constants are correctly defined in your PLC's Tag Database, the ladder logic will automatically use them in its calculations, detecting the crossing of that specific line.

This setup is sophisticated and addresses many aspects of real-world GPS integration in PLCs. It's a solid foundation for your project!

Sure, let's look at this from the perspective of handling multiple rows and a "trigger-cut" action where the TriggerCut engages, completes its cycle, and then resets, awaiting the next manual trigger for the next row. This is a common requirement in agricultural automation.

The core change here is that your previous "line crossing" detection was for one specific line and then latched the TriggerCut ON until a manual reset. Now, you want to:

- Manage multiple "rows" (lines).

- Manually initiate a cut for a specific row.

- Have the TriggerCut cycle (engage, do its thing, reset) automatically.

- Track which rows have been processed.

This requires a more sophisticated state machine or sequence control, combined with your existing line-crossing detection.

Here's a breakdown of how to approach this, including changes to your PLC logic and C-more screen:

### Key Concepts for Multiple Rows and Trigger-Cut:

- Line Database: Instead of just P1 and P2, you'll need an array or structure to store the coordinates of all your rows.

- Row Selection: You'll need a way on the HMI to select which "row" you are currently working on or want to trigger.

- Trigger Input: A dedicated button to initiate the "cut" sequence for the currently selected row.

- TriggerCut Cycle Management: The PLC needs logic to engage the TriggerCut, monitor its progress (e.g., a timer, a limit switch), and then disengage it.

- Row Tracking: A way to mark rows as "completed" to prevent re-triggering on the same row, or to know which ones are left.

### PLC Logic Changes (Productivity Suite):

1. Data Structure for Rows:

Instead of individual Line_P1_X_Meters, etc., create a UDS (User-Defined Structure) or simply use arrays. A UDS is cleaner.

Option A: UDS Structure (Recommended for readability)

- Create a UDS named CropRow_t (or similar):

  - P1_X (FLOAT)

  - P1_Y (FLOAT)

  - P2_X (FLOAT)

  - P2_Y (FLOAT)

  - Triggered (BOOL) - To indicate if this row has been triggered.

  - Completed (BOOL) - To indicate if the TriggerCut cycle for this row is finished.

- Create an Array of this UDS:

  - CropRows[10] (Array of CropRow_t type) - Where 10 is your maximum number of rows.

Option B: Separate Arrays (If UDS is too complex initially)

- Row_P1_X[10] (FLOAT Array)

- Row_P1_Y[10] (FLOAT Array)

- Row_P2_X[10] (FLOAT Array)

- Row_P2_Y[10] (FLOAT Array)

- Row_Triggered[10] (BOOL Array)

- Row_Completed[10] (BOOL Array)

2. Current Row Index:

- Current_Row_Index (INT) - This tag will store the index of the row (e.g., 0, 1, 2...) that you've selected on the HMI or are currently processing.

3. TriggerCut Control Flags and Timers:

- TriggerCut_Manual_Trigger (BOOL) - From HMI button.

- TriggerCut_Engage_Command (BOOL) - The output to lower the TriggerCut (replaces TriggerCut_Control_Output).

- TriggerCut_Active (BOOL) - Status flag: TriggerCut is currently in the ground.

- TriggerCut_Cycle_Complete (BOOL) - Indicates the ditching action for the current row is done.

- TriggerCut_Lower_Time (TIMER) - If the TriggerCut cycle is time-based.

- TriggerCut_Lower_Duration_S (FLOAT Constant) - Configurable TriggerCut lower time.

- TriggerCut_Raised_Limit_Switch (BOOL Input) - If you have a physical sensor to confirm the TriggerCut is fully raised.

- TriggerCut_Lowered_Limit_Switch (BOOL Input) - If you have a physical sensor to confirm the TriggerCut is fully lowered.

4. Modified Rungs:

- Rung X: Update Line_P1/P2 based on Current_Row_Index:

  - This is a new section. When Current_Row_Index changes (or on a GPS_New_Data_Ready_Pulse), you'll copy the P1/P2 values from the selected CropRows array element into temporary tags that your cross-product calculation will use.

  - Use MOV instructions:

Code snippet

// Trigger this on GPS_New_Data_Ready_Pulse OR on a change to Current_Row_Index

|---------------------------------[NO] GPS_New_Data_Ready_Pulse---------------[MOV]-----------------|

|                                 [OR] Row_Index_Changed_Pulse             | Source: CropRows[Current_Row_Index].P1_X |

|                                                                          | Dest: Line_P1_X_Calc | // These are the tags used in cross-product calc |

|--------------------------------------------------------------------------+-----------------------|

// Repeat for P1_Y, P2_X, P2_Y

  - Your cross-product calculations (Rung 10) would then use Line_P1_X_Calc, Line_P1_Y_Calc, etc.

- Rung 11 (Line Crossing Detection - Now triggers Row_Crossed_Pulse):

  - This logic remains similar, but instead of setting Line_Cross_Detected_Pulse, you might set a Row_Crossed_Pulse.

  - Crucial Addition: Add an AND condition to ensure this detection only happens if the selected row hasn't already been triggered/completed: [NO] CropRows[Current_Row_Index].Completed.

  - The detection would set Row_Crossed_Pulse.

- Rung 13 (New): TriggerCut Control State Machine / Sequence:

  - This is a key new section. It handles the TriggerCut_Engage_Command and TriggerCut_Active states.

  - States (Simplified Example):

    - Idle: TriggerCut raised, waiting for trigger.

    - Lowering: TriggerCut engaging.

    - Active: TriggerCut fully lowered, performing cut.

    - Raising: TriggerCut disengaging.

    - Cycle Complete: TriggerCut raised, row marked complete.

  - Logic Example:

Code snippet

// TriggerCut Control Logic

// State 1: Idle - Wait for Manual Trigger AND Row Crossing AND Row Not Completed Yet

|---------------------------------[NO] TriggerCut_Manual_Trigger---------------+-------------------|

|                                 [AND] [NO] Row_Crossed_Pulse            |                   | // Triggered from line crossing detection |

|                                 [AND] [NO] CropRows[Current_Row_Index].Completed |                   | // Ensure row not already done |

|                                                                             |---[SET] TriggerCut_Engage_Command---| // Lower the TriggerCut |

|                                                                             |---[SET] TriggerCut_Active---| // Indicate TriggerCut is active |

|                                                                             |---[SET] CropRows[Current_Row_Index].Triggered---| // Mark row as triggered |

|                                                                             |---[RST] Row_Crossed_Pulse---| // Reset the crossing pulse |

|-----------------------------------------------------------------------------+-------------------|

// State 2 & 3: Monitor TriggerCut Active Time (If time-based)

|---------------------------------[NO] TriggerCut_Active-----------------[TIMER] TriggerCut_Lower_Time-------|

|                                                                     Mode: TOF (Timer Off Delay)       | // or TON, depends on exact timing needed |

|                                                                     Time: TriggerCut_Lower_Duration_S    |

|----------------------------------------------------------------------------------------------------|

// State 4: Disengage TriggerCut when time is up (or limit switch reached)

|---------------------------------[NO] TriggerCut_Lower_Time.DN----------[RST] TriggerCut_Engage_Command---| // If using TON, this would be TriggerCut_Lower_Time.Q |

|----------------------------------------------------------------------------------------------------|

// State 5: Mark Cycle Complete when TriggerCut is fully raised (or time up)

|---------------------------------[NO] TriggerCut_Engage_Command---------+-------------------| // TriggerCut is now raising/raised |

|                                 [AND] [NO] TriggerCut_Active           |                   | // Make sure it's not active anymore |

|                                 [AND] [NO] TriggerCut_Raised_Limit_Switch|                   | // OPTIONAL: If you have a raised limit switch |

|                                                                     |---[SET] CropRows[Current_Row_Index].Completed---| |

|                                                                     |---[RST] TriggerCut_Active---| // Ensure active is off (redundant but good) |

|---------------------------------------------------------------------|-------------------|

  - Refinement: If you have actual limit switches, your logic for TriggerCut_Engage_Command would involve turning on the lowering solenoid, waiting for TriggerCut_Lowered_Limit_Switch to be TRUE, then activating the TriggerCut function. To raise, you'd turn off the lowering solenoid (or activate raising solenoid) and wait for TriggerCut_Raised_Limit_Switch.

- TriggerCut_Control_Output (Physical Output): This should be directly tied to TriggerCut_Engage_Command.

### C-more Screen Changes:

This is where the user interaction for multiple rows comes in.

- Row Selection:

  - Object: Numeric Entry for Current_Row_Index (e.g., "Row #").

  - Object: Increment/Decrement Buttons associated with Current_Row_Index (to easily cycle through rows).

  - Object: Numeric Display showing Current_Row_Index.

- Row Point Input:

  - Object: Four Numeric Entry objects to display/edit CropRows[Current_Row_Index].P1_X, etc.

    - Important: C-more allows you to use indexed tags like CropRows[Current_Row_Index].P1_X. The Current_Row_Index will dynamically change which row's points are displayed/edited.

  - Object: A Pushbutton labeled "Load Row Points" or "Save Row Points".

    - Purpose: To explicitly load the points for the selected Current_Row_Index into the Line_P1_X_Calc (etc.) tags that the line-crossing calculation uses. This prevents unintentional updates if you're just Browse rows.

    - Action: Triggers a PLC bit (Load_Row_Points_PB), and then in the PLC, that bit enables MOV instructions to copy CropRows[Current_Row_Index].P1_X to Line_P1_X_Calc, etc.

- Manual Trigger Button:

  - Object: Momentary Pushbutton for TriggerCut_Manual_Trigger.

  - Text: "Trigger TriggerCut for Selected Row".

  - Action: Sets the TriggerCut_Manual_Trigger bit in the PLC.

- Status Indicators per Row:

  - Object: Pilot Lights or Multi-State Indicators for CropRows[Current_Row_Index].Triggered and CropRows[Current_Row_Index].Completed. This helps you see the status of the currently selected row.

  - Optional (Advanced): If you want to see the status of all rows on one screen, you'd need a more complex "recipe" or "data block" display, or simply create many indicators (Row1_Triggered, Row2_Triggered, etc.).

- TriggerCut Status:

  - Object: Pilot Light for TriggerCut_Active.

### Overall Workflow:

- Preparation (Offline/Setup):

  - You survey all your crop rows.

  - You convert all P1_Lat/Lon and P2_Lat/Lon for each row into P1_X/Y_Meters and P2_X/Y_Meters relative to your Local_Ref_Lat/Lon.

  - You either manually enter these into the CropRows array tags in the PLC software, or you use the C-more HMI to enter them.

- At the Field:

  - Start the tractor and PLC/HMI.

  - On the C-more screen, use the "Row #" selector to choose the row you're about to approach (e.g., Row 1).

  - Press the "Load Row Points" button to make sure the PLC is actively looking for Row 1.

  - Drive the tractor towards Row 1.

- Trigger and Cut:

  - When you are positioned correctly and ready to cut at Row 1, press the "Trigger TriggerCut for Selected Row" button on the HMI.

  - The PLC then waits for the Row_Crossed_Pulse for that specific row's coordinates.

  - Once the tractor crosses Row 1 (and the GPS signal is valid, and the row hasn't been completed yet), the PLC logic will:

    - Engage the TriggerCut (TriggerCut_Engage_Command).

    - Start the TriggerCut cycle (e.g., timer for duration).

    - Mark CropRows[Current_Row_Index].Triggered to TRUE.

  - After the TriggerCut cycle completes (timer done / limit switch active), the PLC will:

    - Disengage the TriggerCut.

    - Mark CropRows[Current_Row_Index].Completed to TRUE.

- Next Row:

  - Drive to Row 2.

  - On the HMI, change "Row #" to 2.

  - Press "Load Row Points" again.

  - Press "Trigger TriggerCut for Selected Row".

  - Repeat the process.

This setup gives you the flexibility to manage multiple rows and precisely control when the ditching action occurs for each. It's a significant step up from a single-line, continuous shutdown.

This is an excellent thought process! You're correctly identifying that the relative straightness and parallel nature of crop rows simplifies things. Yes, using the y=mx+b concept for parallel lines with similar slopes is a very common and efficient way to handle this in real-world agricultural automation.

Let's break down how to handle multiple rows with mostly parallel lines, focusing on efficiency and flexible detection.

### The Challenge with y=mx+b Directly in PLC & The Solution

You're right that storing m and b for each row makes sense. However, the y=mx+b form has the inherent issue of an infinite slope for vertical lines (where X is constant). While your crop rows might be mostly parallel and have a consistent slope, the line the tractor crosses might be oriented in any direction.

The cross-product method we've been using, which takes two points for the line, is inherently robust for any line orientation (horizontal, vertical, diagonal) without special cases. It's often simplified to a constant, C, and you check the sign of Ax+By−C.

For parallel lines, a more effective approach is to define a "base" line (your first row, or a reference line), get its slope, and then use the perpendicular distance from the tractor to that line, comparing it to predefined "offsets" for each row.

### Preferred Method: Perpendicular Distance from a Reference Line

Given that your rows are relatively straight and parallel, you can define them by:

- A single reference line (e.g., your first crop row). You'd define this line with its two points, RP1 and RP2.

- A series of perpendicular offset distances for each subsequent row from that reference line.

This means all rows share the same 'slope' but have different 'intercepts' (or 'C' values in the general line equation).

General Line Equation:

Any straight line can be represented by the equation:

Ax+By+C=0

Where (A,B) is a vector perpendicular to the line. For parallel lines, A and B will be the same for all rows, and only C will change.

How to get A, B, and C for your base line (from RP1, RP2):

- Calculate vector V=(RP2_x−RP1_x,RP2_y−RP1_y). This vector points along your base line.

- A vector perpendicular to V is N=(−V_y,V_x) or (V_y,−V_x). Let's use N=(−(RP2_y−RP1_y),(RP2_x−RP1_x)). So, A=−(RP2_y−RP1_y) and B=(RP2_x−RP1_x).

- Calculate C using one of the points on the line (e.g., RP1): C=−(AcdotRP1_x+BcdotRP1_y).

  - So, C=−((−(RP2_y−RP1_y))cdotRP1_x+((RP2_x−RP1_x))cdotRP1_y)

This derivation can be tricky for the PLC. So, instead of deriving A, B, C directly, we can leverage the cross product itself.

### Simplified Approach: Using Cross Product Magnitude for Perpendicular Distance

The cross product (P_x−A_x)(B_y−A_y)−(P_y−A_y)(B_x−A_x) gives a value proportional to the signed perpendicular distance from point P to the line defined by A and B.

Let's call the value of this cross product Line_Deviation_Value.

Line_Deviation_Value = (Veh_Current_X_Meters - RP1_x)(RP2_y - RP1_y) - (Veh_Current_Y_Meters - RP1_y)(RP2_x - RP1_x)

The actual perpendicular distance D is abs(Line_Deviation_Value) / sqrt((RP2_x - RP1_x)^2 + (RP2_y - RP1_y)^2).

However, we don't need the actual distance in meters, just a consistent value that represents how far you are from the line, and on which side. Line_Deviation_Value (the cross product) serves this purpose perfectly.

Here's the refined strategy for multiple, mostly parallel rows:

- Define ONE Base Reference Line:

  - You'll still have Line_Ref_P1_X_Meters, Line_Ref_P1_Y_Meters, Line_Ref_P2_X_Meters, Line_Ref_P2_Y_Meters. These define your first physical crop row (or a theoretical starting line).

  - These would be FLOAT constants in your PLC.

- Define Row Offsets:

  - Create an array: Row_Offsets_CrossProduct[Number_of_Rows] (FLOAT Array).

  - For each row N, this value will be the target cross product value that represents that specific row.

  - How to get these values?

    - Drive the tractor precisely along your first row. Note down the Line_Deviation_Value for this line. This will be Row_Offsets_CrossProduct[0]. (It should ideally be close to 0 if RP1/RP2 are on the line).

    - Drive the tractor precisely along Row 2. Calculate Line_Deviation_Value using Line_Ref_P1/P2 and your current tractor position. This value will be Row_Offsets_CrossProduct[1].

    - Repeat for all rows.

    - Alternatively: You can calculate this by taking Line_Ref_P1/P2, then manually finding a point (X_rowN, Y_rowN) that lies on row N. Then calculate (X_rowN - RP1_x)(RP2_y - RP1_y) - (Y_rowN - RP1_y)(RP2_x - RP1_x) for each row. This will give you the offset value.

- Calculate Current Perpendicular Deviation:

  - In your PLC, continuously calculate Current_Cross_Product_Deviation = (Veh_Current_X_Meters - Line_Ref_P1_X_Meters) * (Line_Ref_P2_Y_Meters - Line_Ref_P1_Y_Meters) - (Veh_Current_Y_Meters - Line_Ref_P1_Y_Meters) * (Line_Ref_P2_X_Meters - Line_Ref_P1_X_Meters)

  - This single value tells you where the tractor is relative to your base reference line.

- Identify Which Row is Being Crossed:

  - You'll need to compare Current_Cross_Product_Deviation with the values in Row_Offsets_CrossProduct[].

  - You'll define a small Tolerance_Deviation (FLOAT constant, e.g., 0.5 for 0.5 meters equivalent deviation, you'll need to calibrate this to your cross-product scale).

  - For each row i, you are "on" row i if:

    - (Current_Cross_Product_Deviation > (Row_Offsets_CrossProduct[i] - Tolerance_Deviation))

    - AND (Current_Cross_Product_Deviation < (Row_Offsets_CrossProduct[i] + Tolerance_Deviation))

  - This gives you a BOOL for each row, e.g., On_Row[i].

- Tracking Row Status and Triggering:

  - Row Status: Row_Processed[Number_of_Rows] (BOOL Array). Initialize all to FALSE.

  - Trigger Input: TriggerCut_Manual_Trigger (BOOL from HMI).

  - Active Row Index: Active_Row_Index (INT) - identifies the row you just triggered.

PLC Logic Flow (Simplified Rungs):

Code snippet

// Rung 1: Calculate Current_Cross_Product_Deviation

// ... (logic from section 3 above) ...

// Rung 2: Identify On_Row[i] (for all 'i')

// For each row i:

|---------------------------------[NO] GPS_Signal_Valid_Flag------------[CMP]-------------------|

|                                 [AND]                                 Input1: Current_Cross_Product_Deviation |

|                                                                       Input2: Row_Offsets_CrossProduct[i] - Tolerance_Deviation |

|                                                                       Operator: Greater Than |

|                                 -------------------------------------[CMP]-------------------|

|                                                                       Input1: Current_Cross_Product_Deviation |

|                                                                       Input2: Row_Offsets_CrossProduct[i] + Tolerance_Deviation |

|                                                                       Operator: Less Than |

|-----------------------------------------------------------------------[OUT] On_Row[i]--------|

// Rung 3: Process TriggerCut Trigger

// This is the core 'trigger-cut' logic. You need a trigger button from the HMI.

|---------------------------------[NO] TriggerCut_Manual_Trigger_PB----------+-------------------| // Your HMI button

|                                 [AND] [ONE-SHOT] TriggerCut_Manual_Trigger_PB |                   | // Detect button press edge |

|                                                                             |                   |

|  // Find which row is currently "On_Row" and not yet processed             |                   |

|  // This requires a loop or cascaded logic if you don't have a dedicated "find first true" instruction |

|  // For simplicity, let's assume we find the lowest index 'i' where On_Row[i] is true and Row_Processed[i] is false |

|  // Let's call the determined index 'Trigger_Candidate_Row_Index' (This is the trickiest part of the logic without a loop/FOR instruction) |

|  // In Productivity, you might use a sequence of `MOV` and `CMP` to find it, or a `FOR` loop if your version supports it well for this. |

|                                                                             |                   |

|                                                                             |---[SET] TriggerCut_Engage_Sequence_Start---| // Start the TriggerCut sequence |

|                                                                             |---[MOV] Trigger_Candidate_Row_Index---| // Store which row we're processing |

|                                                                             |     Active_Row_Index          |   |

|-----------------------------------------------------------------------------+-------------------|

// Rung 4: TriggerCut Sequence (similar to previous 'TriggerCut control' states, but now uses Active_Row_Index)

// This sequence needs to SET the Row_Processed[Active_Row_Index] to TRUE when it completes.

// TriggerCut_Engage_Sequence_Start will trigger your timer/limit switch logic.

// Once complete, it will SET Row_Processed[Active_Row_Index] = TRUE and RST TriggerCut_Engage_Sequence_Start.

Handling "Which row comes first" (Approaching Rows):

This is the tricky part without explicitly telling the PLC which row you're targeting.

- Option A: Manual Selection (As discussed previously)

  - You pick Current_Row_Index on the HMI.

  - The PLC only looks for a crossing of that specific row when you press the trigger.

  - Pros: Simplest to implement. Explicit control.

  - Cons: Requires operator interaction for each row.

- Option B: Automatic Nearest Unprocessed Row (More Complex)

  - The PLC continuously calculates Current_Cross_Product_Deviation.

  - It then iterates through all Row_Offsets_CrossProduct[] values.

  - It finds the Row_Offset that is closest to Current_Cross_Product_Deviation AND whose corresponding Row_Processed[i] flag is FALSE.

  - This closest unprocessed row becomes your Trigger_Candidate_Row_Index.

  - When you press TriggerCut_Manual_Trigger_PB, it then triggers the TriggerCut sequence for this Trigger_Candidate_Row_Index.

  - Implementation Difficulty: This requires more advanced PLC logic (potentially FOR loops or complex CMP cascades to find the minimum absolute difference among unprocessed rows). Productivity has "Find" instructions and "FOR" loops, but they need to be used carefully within the scan cycle.

  - Pros: Less manual selection on the HMI.

  - Cons: More complex PLC code. Risk of selecting the "wrong" row if rows are very close or GPS signal jumps.

- Option C: Sequential Processing (Assumes you go in order)

  - You have a Next_Row_To_Process_Index tag.

  - When you press the trigger button, the PLC only looks for a crossing of Row_Offsets_CrossProduct[Next_Row_To_Process_Index].

  - Once that row is processed and Row_Processed[Next_Row_To_Process_Index] becomes TRUE, you then INC (increment) Next_Row_To_Process_Index.

  - Pros: Simple, deterministic.

  - Cons: Requires strict adherence to processing rows in order. If you skip a row or go back, it breaks the sequence.

### Recommendation:

Given your current comfort level and the complexity introduced by automatic row detection:

Start with Option A: Manual Selection of Current_Row_Index on the C-more HMI.

This is the most robust and easiest to debug for your initial implementation. The operator selects "Row 5", presses "Trigger", and the PLC is specifically looking to cross the Line_Deviation_Value target for Row 5. Once that cut is done, you then select "Row 6" and repeat.

If you master this, and genuinely need more automation, then explore Option B or C. Option B, while ideal, can be quite challenging to implement perfectly in ladder logic without strong "array processing" or "looping" features, especially if you need to find the closest unprocessed row dynamically.

The key mathematical shift is from defining each line with two points to defining a reference line with two points and then all other lines by their perpendicular offset value from that reference line. The cross-product value itself is that consistent perpendicular deviation.

Introduction

Motivation:

Interbody fusion devices (IFDs) can be damaged during insertion into the spinal disc space which occurs via malleting by the surgeon. It is uncertain which aspect of the insertion process causes IFD failure due to variations in malleting (Palepu et al., 2021).

The Mississippi Delta faces critical water challenges, with declining groundwater in the Mississippi River Valley Alluvial Aquifer demanding sustainable irrigation. While modern tools optimize water plans, their field-level effectiveness is limited. Efficient water use is crucial for crop productivity and environmental stewardship in our agricultural heartland.

Challenge:

Currently, there is little to no data on the loading conditions of the instruments used to insert IFDs during transforaminal lumbar interbody fusion (TLIF) procedures that compares studies conducted using a benchtop device to the use of cadavers (Kienle et al., 2016).

Currently, manually punching holes in lay-flat poly irrigation tubing is standard practice. This leads to inconsistent spacing, and variable diameters. Such irregularities cause uneven water distribution, reduced efficiency, and pipe deformation, undermining precisely designed irrigation strategies and increasing labor demands.

Solution:

The goal of this research was to develop a benchtop setup capable of recreating the impact waveform generated during impaction in cadaver studies to replace the use of cadavers in this scope of testing. Four waveform features were selected for mimicking the cadavers’ impact waveforms: (i) peak force, (ii) slope of the impact wave, (iii) area under the curve, and (iv) impact duration.

Our project introduces an Automated Hole Insertion System for lay-flat irrigation tubing. This system integrates precision mechanical components, encoder feedback, and PLC-based control. It ensures highly consistent hole spacing and superior hole quality, significantly improving irrigation uniformity, reducing labor, and enhancing overall water-use efficiency for sustainable agriculture.

Methods

Benchtop Device:

- A benchtop device was developed and utilized a piezoelectric impact force sensor, film force sensor, and a laser displacement sensor to quantify the impact of a drop weight onto the insertion tool.

- The IFD was located on the end of the insertion tool and wedged between Sawbones bone foam attached to vertical platens loaded with a compressive force to simulate vertebrae.

- The insertion tool was impacted by the drop weight until the IFD was fully inserted between the platens.

Cadaver Study:

- A full cadaver study using two different cadavers was conducted to validate the benchtop device.

- The insertion tool used was outfitted with the same sensors used on the benchtop device.

Benchtop Study:

- The experimental design included all 18 possible combinations of testing parameters for drop weight (0.50, 0.75, and 1.00 lb), drop height (20, 40, and 60 cm), and interbody fusion device size (12 and 14 mm height with 24 mm width).

- The compression platens used to simulate the vertebrae were set apart at a distance such that an estimated 200 N lateral compressive load was applied with respect to the 12 mm device at full insertion.

- For each testing group, a separate device was fully inserted 8 times using the drop weight impact device (n=8 insertion tests per device/condition). Full insertion was defined as 1.5 times the length of the device.

- Throughout each insertion test, the lateral compressive load and displacement of the device were recorded.

Conclusions:

- This work validated the capability of a benchtop device to accurately quantify and replicate the peak force waveform and slope of the impact wave of cadaveric model.

- For peak force, the 12 mm interbody device size, 60 cm drop height, and both the 0.75 and 1.0 lb drop weights replicated the peak forces measured from the cadaveric model.

- When comparing positive slope of each benchtop group to that of the cadaver group, no significant difference was observed for several benchtop conditions.

Future Work:

- Currently, the benchtop device is undergoing modifications to more accurately mimic insertion of an IFD into a spinal disc space.

- Further testing will be conducted with varying drop weight materials and compressive loads to attempt to more accurately mimic cadaveric waveforms of area under the curve and impact duration.