How to Use GPS

Perfect, Micah — Option 2 with vector geometry is exactly the right tool for this kind of situation, especially since you’ll be crossing angled lines like crop rows at unpredictable paths.

Let’s break this down into a clear plan you can implement in your PLC logic or simulate first in Python. I’ll walk you through the concept first, then the math, then how to code it.

## ✅ GOAL

Determine if your GPS position crosses a line segment between Point A and Point B, regardless of the line’s orientation.

You’ll know:

- The GPS position at time t₁: P_prev = (lat₁, lon₁)

- The GPS position at time t₂: P_now = (lat₂, lon₂)

- The line segment you’re monitoring: from A = (lat_a, lon_a) to B = (lat_b, lon_b)

## 📐 Method: Line Segment Intersection via 2D Vectors

You’re checking if the segment P_prev → P_now intersects the segment A → B.

### Step 1: Convert Coordinates into 2D Vectors

- Treat latitude as Y and longitude as X.

- So points become:

- A = (x1, y1)   # Start of the line

- B = (x2, y2)   # End of the line

- P_prev = (x3, y3)

- P_now = (x4, y4)

### Step 2: Use the Line Segment Intersection Formula

You can use the orientation method or cross product. Here’s the best approach for this:

#### 🔧 Compute Orientation

For three points p, q, r, the orientation is:

val=(qy−py)(rx−qx)−(qx−px)(ry−qy)\text{val} = (q_y - p_y)(r_x - q_x) - (q_x - p_x)(r_y - q_y)

- If val > 0: clockwise

- If val < 0: counterclockwise

- If val = 0: colinear

### Step 3: Apply it to All Combinations

To detect intersection of segments AB and P_prev→P_now:

- Let’s name them:

  - Line 1: A-B

  - Line 2: P_prev → P_now

You need to compute:

o1 = orientation(A, B, P_prev)

o2 = orientation(A, B, P_now)

o3 = orientation(P_prev, P_now, A)

o4 = orientation(P_prev, P_now, B)

And then check:

if o1 != o2 and o3 != o4:

# You crossed the line!

## 🧠 Coordinate Format Note

You’ll need to:

- Convert GPS strings to decimal degrees

- Optionally convert those degrees to feet or meters if you want consistent units for distance

Want help with a small conversion block for this in feet/meters?

## 🔄 Continuous Detection Logic (Live GPS Feed)

Every update:

- Store last GPS reading (P_prev)

- Get current GPS (P_now)

- Check for intersection between P_prev → P_now and any monitored line A → B

- If yes → trigger output

You can even check against multiple line segments this way to monitor for row changes or section crossings.

## ⚙️ Want a Working Example?

I can provide a:

- ✅ Python function you can simulate this with

- ✅ Structured Text (ST) or pseudo-ladder logic block for Productivity Suite

Which would you prefer to start with? Python is best for visualizing/debugging, and once it works, I can help port it to your PLC.

How to go from ArcGIS → GPS map → PLC triggers, step by step:

## 🗺️ STEP 1: Create Field Maps in ArcGIS

In ArcGIS Pro or ArcGIS Online:

- Plot your field lines (boundaries, rows, trigger zones, etc.).

- Export them as shapefiles (.shp) or GeoJSON / CSV with coordinates.

- For each line or area, include attributes (e.g., FieldName, TriggerID, ActionType) to identify them later.

You now have GPS-coordinate-based vector lines you can use for logic.

## 🔄 STEP 2: Convert ArcGIS Data to a Format Usable by the PLC

The Productivity2000 PLC can’t read .shp or .geojson files directly, so here’s the middle-layer approach:

### ✔️ Recommended Workflow:

| Tool | Purpose |
| --- | --- |
| 🛰 ArcGIS | Plot lines and export coordinates |
| 💻 Python / PC app | Preprocess data, format it |
| 💽 SD card or serial | Transfer simplified coordinate lists to PLC |
| 🤖 PLC | Use logic to compare real-time GPS to map lines |

### 💡 Convert your line data to a CSV or register-based list:

LineID,LatA,LonA,LatB,LonB,TriggerID

1,34.30123,-88.42321,34.30123,-88.42111,100

2,34.29987,-88.42319,34.29987,-88.42109,101

...

## 🧠 STEP 3: PLC Logic to Read and Use Map Data

Once you’ve loaded line segments into the PLC (manually, from SD card, or over serial), the PLC does the real-time GPS checking, just like we discussed:

- For each line:

  - Compare GPS vector (P_prev → P_now) with stored segment (A → B)

  - Use intersection logic (cross-product or orientation method)

- If crossed → Trigger the corresponding TriggerID output

- Use the TriggerID to run logic or display something on the HMI (C-More)

## 🖥️ STEP 4: Use C-More HMI to Load Maps or Monitor Progress

The C-More HMI can:

- Display a dropdown list of available fields/maps (Field A, Field B, etc.)

- Trigger a PLC tag that loads a specific line set into memory

- Show active GPS coordinates, status messages, or zone info

- Show logs of which field/line was crossed and when

Optional: Store multiple field line-sets in the PLC memory or on an SD card, and load the correct one based on a user selection in the HMI.

## 🚜 Example Use Case

Let’s say your system is driving through a rowed field. You could:

- Map each crop row line in ArcGIS

- Export all lines for that field as a CSV with Lat/Lon

- Load the appropriate field lines into the PLC

- Continuously monitor GPS updates from StarFire 6000

- Detect when a row is entered or exited

- Automatically log or trigger actions (e.g., apply fertilizer, open valve, log data, etc.)

## 🔧 Next Steps

Here’s what I can help you with next:

- ✅ Python script to convert ArcGIS line exports to PLC-ready CSV

- ✅ Structured Text or Ladder logic to read a line segment and compare it to GPS position

- ✅ SD card reading logic or serial comms (if you want to dynamically load maps into the PLC)

- ✅ C-More screen logic to select a field and monitor trigger status

Would you like to start with the Python conversion script and a PLC logic block to load and compare lines?