PolyPipe Device Hardware/Code Protocol

By: Micah Foster

Necessary Equipment:

- Productivity2000:

  - P2-550 CPU

  - P2-04AD

  - P2-08TRS

  - P2-08SIM

- Strider (SE2-SW5UG-T)

- C-more EA9 series touch screen HMI (EA9-T10WCL)

- AMC4 connected with:

  - Encoder

  - Stepper motor 1 + Encoder

  - Stepper motor 2

  - Stepper motor + Encoder

  - SureMotion linear actuator assembly

  - Roller Encoder

  - Connecting AutomationDirect photoelectric fork sensor:

  - Gefran potentiometer linear position sensor

- SureStep AC microstepping stepper drive (STP-DRVAC-24025)

- SureStep unregulated linear power supply (stp-pwr-3204)

- AC/DC Din Rail power supply (24 W, 24 VDC, 1.0 A)

Wiring Device:

- Productivity2000:

  - P2-550 CPU

  - P2-04AD

    - Location: 4 slot base board

    - Task:** Gathers data from Roller Encoder

  - P2-08TRS

  - P2-08SIM

- Strider (SE2-SW5UG-T)

- AMC4 connected with:

  - Encoder

  - Stepper motor 1 + Encoder

    - Location: Bottom of the device

    - Task: Rotate the cutter die at the same velocity of material moving across the platform (rotate only when cut needed)

    - Connected to:** AMC-1-axis-1

  - Stepper motor 2

    - Location: Top of the device

    - Task: Match the velocity of the polypipe (constant rotation)

    - Connected to: AMC-1-Axis-2

  - Stepper motor + Encoder

    - Location: Side of platform

    - Task: Move platform horizontally to material

    - Connected to: AMC-1-Axis-3

  - SureMotion linear actuator assembly

  - Roller Encoder

    - Location: Connected to roller that material turns

    - Task: Gather velocity of material

  - Connecting AutomationDirect photoelectric fork sensor:

    - (+) Wire: P2-04AD 24V+ port

    - Signal wire: AMC-1-Axis-1 port 7 (IN3 Home), known in program as PolyPipe.Ax1In3

  - Gefran potentiometer linear position sensor