Program: Polypipe V.2.062424

Hardware:

- productivity2000

- P2-550 CPU

  - P2-04AD

    - Location: 4 slot base board

  - P2-08TRS

  - P2-08SIM

- Strider (SE2-SW5UG-T)

- Amc4 connected with

  - Encoder (TRDA-20R1N2500VD)

  - Stepper motor 1 + Encoder

    - Location: bottom of the device

    - Task: rotate the cutter die at the same velocity of material moving across the platform (rotate only when cut needed)

    - Connected to: AMC-1-axis-1

  - Stepper motor 2

    - Location: top of the device

    - Task: match the velocity of the polypipe (constant rotation)

    - Connected to: AMC-1-Axis-2

  - Stepper motor + Encoder

    - Location: side of platform

    - Task: move platform horizontally to material

    - Connected to: AMC-1-Axis-3

    - SureMotion linear actuator assembly

- Roller Encoder

  - Location: connected to roller that material turns

  - Task: gather velocity of material

- Connecting AutomationDirect photoelectric fork sensor: (+) wire to P2-04AD 24V+ port. Signal wire to AMC-1-Axis-1 port 7 (IN3 Home), known in program as PolyPipe.Ax1In3

- Gefran potentiometer linear position sensor

- Stepper Driver: STP-DRVAC-24025

- Linear Actuator: LAHP-25TM102LP25

Software:

Rung 1: a normally open contact (NO) that says "SystemEnable"  at the end of the line i have the word "enable" then a Velocity Move function named 'top motor' looking at the velocity move setup it shows axis name: Top Motor which is this device “Stepper motor 2” it is using a ‘Use Structure’ and the name is Top Motor. Move setup: direction = negative. Stop setup: stop at maximum decel rate.

Rung 2: normally open contact (NO) that is titled “SystemEnable”. At the end of the line there is a simple move function. On the simple move function we see: module name: amc-1, channel/axis: 1, channel axis name: Die Motor, move addressing: absolute, direction: negative, target: 1, velocity: PolyPipe.Ax4Velocity, acceleration: DieAccelDecelRate, Deceleration: DieAccelDecelRate, Stop Setup: immediate stop.

Rung 3: (NO) contact: “Home_Die_Motor”, (SET) set coil: “LatchHomeDie”.

Rung 4: (NO) contact: “LatchHomeDie”, (NO) contact: Polypipe.ax1in3, (RST) Reset Coil: “LatchHomeDie”.

Rung 5: (NO) contact: “LatchHomeDie”, (NC) Normally Closed Contact: SystemEnable, [Find Home function: Module Name = Amc-1, Channel/Axis = 1, Channel/Axis Name = Die Motor, Mode = 2-moce to switch 1 and halt, init direction = positive, Switch 1 input = In3-Home, Edge = Rising]

Rung 6: (NO) contact: “LatchHomeDie”, (NC) Normally Closed Contact: SystemEnable, [Velocity Move: Amc-1, Channel/Axis = 2, Channel/axis name = Top Motor, Move Direction = Negative].

Rung 7: (NO) contact: “Home_Table_Motor”, (SET) coil: “LatchHomeTable”,

Rung 8: (NO) contact: “LatchHomeTable”, Compare Contact (CMP): “PotentiometerPositionMM” < “Potent_Edge_scale_mm”, RST coil: “LatchHomeTable”,

Rung 9: (NO) contact: ““LatchHomeTable”, Compare Contact (CMP): “PotentiometerPositionMM” > “Potent_Edge_scale_mm”, (NC) contact: “SystemEnable”, Velocity move: Module = AMC-1, Channel/axis = 3, channel/axis name = table motor, move direction = negative,

Rung 10: (NO) contact: “Home_Table_Motor”, (SET) coil: “LatchHomeTable”,

Rung 11: (NO) contact: “LatchHomeTable”, Compare Contact (CMP): “PotentiometerPositionMM” > “Potent_Edge_scale_mm”, RST coil: “LatchHomeTable”,

Rung 12: (NO) contact: ““LatchHomeTable”, Compare Contact (CMP): “PotentiometerPositionMM” < “Potent_Edge_scale_mm”, (NC) contact: “SystemEnable”, Velocity move: Module = AMC-1, Channel/axis = 3, channel/axis name = table motor, move direction = positive,

Rung 13: (NO) contact: “SystemEnable”, Compare Contact (CMP): “PotentiometerPositionMM” > “Die Index Target”, Compare Contact (CMP): “CV” > “25”, [Velocity Move: AMC-1, channel/axis = 3, channel/axis name = table motor, move direction = negative, velocity = table motor velocity, ramp rate = table motor ramp rate, stop setup = immediate stop.

Rung 14: (NO) contact: “SystemEnable”, Compare Contact (CMP): “PotentiometerPositionMM” < “Die Index Target”, Compare Contact (CMP): “CV” > “25”, [Velocity Move: AMC-1, channel/axis = 3, channel/axis name = table motor, move direction = positive, velocity = table motor velocity, ramp rate = table motor ramp rate, stop setup = immediate stop.

Rung 15: Math function (MATH): result = Table Motor Velocity, Formula = abs(‘Die Index Target’ – ‘PotentiometerPositionMM’)*5.

Rung 16: MATH function: Result = Table Motor Ramp Rate, Formula = ‘Table Motor Velocity’ *1.2*5.

Rung 17: NC contact: ‘DN’, [Simple Timer: Present Value = 150, Current Value = CV, Done = DN, Time Base = 0.001].

Rung 18: NO Edge Contact (NOE): Polypipe.ax1.In3 (Rising Edge), Set Position: Module name = Amc-1, channel/axis = 1, channel/axis name: die motor, set position to 0.

Rung 19: On column 2 = everything in brackets [NO Edge Contact (NOE): Polypipe.ax1.In3 (Rising Edge), (rung19.1) NO Edge Contact (NOE): SystemEnable (Rising Edge)], SET coil: ‘StopCutCycle’.

Rung 19.1: column 11 = RST coil: TriggerCut,

Rung 19.2: column 4 = NO Edge Contact (NOE): SystemEnable (Rising Edge), column 11 = Set Position: Module name = amc-1, channel/axis = 4, channel/axis name = encoder, set position to 0.

Rung 20: NO contact: ‘TriggerCut’, RST coil: ‘StopCutCycle’.

Rung 21: Math Editor (MATH): result = ‘TriggerPosition’, Formula = ‘PolyPipe.ax4EncPosition’/29.50703.

Rung 22: NO contact: ‘TriggerCut’, NC contact: ‘StopCutCycle’, NO contact: ‘SystemEnable’, Gear Drivetrain: Module name=amc1, drivetrain segments = 1, encoder as primary master axis = yes, primary (master=encoder, ratio=40.3299), Slave=die motor.

Rung 23: NO contact: ‘SystemEnable’, Drivetrain: Module name=amc1, drivetrain segments = 1, encoder as primary master axis = yes, primary (master=encoder, ratio=-40.3299), Slave=Top Motor.

Rung 24: Scale Non-linear function: Input=’Edge Sensor Analog’, Output=’Edge Position MM’

Rung 25: Scale Linear function: input=PolyPipe.ax2EncPosition, Output=’Cutting Motor Degrees’

Rung 26: MATH: Result = ‘DieAccelDecelRate’, Formula = ‘PolyPipe.Ax4Velocity’^2.

Rung 27: MATH: Result = ‘Die Position Target’, Formula = ‘Die Position Capture’ + 400.

Rung 28: MATH: Result = ‘Die Index Target’, Formula = ‘Hole Size’ + ‘Potent_Edge_scale_mm’.

Rung 29: On column 1 [ NO Edge Contact (NOE): Polypipe.ax1.In3 (Rising Edge)], Copy Data: Source = ‘Edge Position MM’, Destination = ‘Edge Position Reference Zero’

Rung 29.1: on column 1 [NO contact: ‘TestEdgePosZero’].

Rung 30: MATH: Result = ‘Die Index Velocity’, Formula = abs(‘PolyPipe.Ax3Position’-‘Die Index Target’).

Rung 31: MATH: Result = ‘Die Index Ramp Rate’, Formula = ‘Die Index Velocity’^2.

Rung 32: Scale Linear: ‘Die Table Position Analog, Output: PotentiometerPositionMM’

Rung 33: Scale Linear: ‘Edge Position MM’, Output = ‘Potent_Edge_scale_mm’.

I am trying to build a code in automation direct ladder logic which will take a value (Numeric Entry) for 'holesize1' and 'Row_Counts' from c-more panel. this will be a numeric tag. this information will tell my software what hole size i should cut for x number of 'Row_Counts'. Once the first set of HoleSize1 has been used for all of the 'Row_counts' number, that action should stop and move to the next action which will be doing the same thing for 'HoleSize2' and 'Row_counts2'. this will continue until we finish HoleSize5 and Row_counts5. I was wanting to use the DRUM function. but i would need to use the 'events' feature which will only take a boolean tag therefore it won't directly correlate with the 'Row_Counts' tag which is a numerical tag. how can i use a for loop to help me accomplish the overall goal.