Program: Polypipe V.2.0

Hardware:

- productivity2000

- P2-550 CPU

  - P2-04AD

    - Location: 4 slot base board

    - Task: gathers data from Roller Encoder

  - P2-08TRS

  - P2-08SIM

- Strider (SE2-SW5UG-T)

- Amc4 connected with

  - Encoder

  - Stepper motor 1 + Encoder

    - Location: bottom of the device

    - Task: rotate the cutter die at the same velocity of material moving across the platform (rotate only when cut needed)

    - Connected to: AMC-1-axis-1

  - Stepper motor 2

    - Location: top of the device

    - Task: match the velocity of the polypipe (constant rotation)

    - Connected to: AMC-1-Axis-2

  - Stepper motor + Encoder

    - Location: side of platform

    - Task: move platform horizontally to material

    - Connected to: AMC-1-Axis-3

    - SureMotion linear actuator assembly

- Roller Encoder

  - Location: connected to roller that material turns

  - Task: gather velocity of material

- Connecting AutomationDirect photoelectric fork sensor: (+) wire to P2-04AD 24V+ port. Signal wire to AMC-1-Axis-1 port 7 (IN3 Home), known in program as PolyPipe.Ax1In3

- Gefran potentiometer linear position sensor

Software:

Rung 1: a normally open contact (NO) that says "Top Motor On"  at the end of the line i have the word "enable" then a Velocity Move function named 'top motor' looking at the velocity move setup it shows axis name: Top Motor which is this device “Stepper motor 2” it is using a ‘Use Structure’ and the name is Top Motor. Move setup: direction = negative. Stop setup: stop at maximum decel rate.

Rung 2: normally open contact (NO) that is titled “Cutting Motor On”. At the end of the line there is a simple move function. On the simple move function we see: module name: amc-1, channel/axis: 1, channel axis name: Die Motor, move addressing: absolute, direction: negative, target: 1, velocity: PolyPipe.Ax4Velocity, acceleration: DieAccelDecelRate, Deceleration: DieAccelDecelRate, Stop Setup: immediate stop.

Rung 3: normally open contact (NO) that is titled “Table Motor On”. At the end of the line there is a simple move function. On the simple move function we see: module name: amc-1, channel/axis: 3, channel axis name: amc-1-axis-3, move addressing: relative, direction: use target sign, target: Die Position.target, velocity: 2, acceleration: 4, Deceleration: 4, Stop Setup: immediate stop.

Rung 4: normally open contact (NO) that is titled “TestHomeSwitch”. At the end of the line there is a set position function. In that function we have as follows: Module Name: AMC-1, Channel/axis:1, channel/axis name: Top Motor, set position to: 0,

Rung 5: on column 3 = NO Edge Contact (NOE) named PolyPipe.Ax1In3 with Rising Edge (on when bit turns on) selected, column 11 = Set Coil named StopCutCycle

Rung 5.1: drops down from rung 5 at column 4, on column 11 = Reset Coil named TriggerCut

Rung 5.2: drops down from rung 5 at column 4, on column 11 = Set Position (Axis/Channel Name: Encoder, Module Name: AMC-1, Channel/Axis: 4, Set Position to: 0)

Rung 6: normally open contact that is titled “TriggerCut” at the end of the line we have a reset coil function (RST) named StopCutCycle.

Rung 7: Math function. With the following setup. Result: TriggerPosition, formula: polypipe.ax4encposition / 29.50703.

Rung 8: Compare contact (CMP) function. With “triggerposition” on the left side of the function and “row spacing inches” on the right side of the function, set to = one another as such: ‘TriggerPosition = Row Spacing Inches’. The line ends with a Set Coil (SET) function named “TriggerCut”

Rung 9: Compare Contact (CMP) function: TriggerPosition > Row Spacing Inches, Math (MATH) function at column 11: Result = TriggerPosition, Formula = TriggerPosition – Row Spacing Inches.

Rung 10: (NO) function named “TriggerCut”, a Normally Closed Contact (NC) named “StopCutCycle”. And ends with a GEAR drivetrain function. The gear drivetrain is set up as such: Module Name: AMC-1, Drivetrain Segments: 1, Encoder as Primary Master Axis: Yes, Primary = (Master: Amc-1axis-4, Slave AMC-1axis-1, Ratio 40.3299, disable)

Rung 11: (NO) function named “Drivetrain Enable”, Gear Drivetrain function setup as such: Module Name: AMC-1, Drivetrain Segments: 1, Encoder as Primary Master Axis: Yes, Primary = (Master: Encoder, Slave: Die Motor, Ratio -40.3299, disable)

Rung 12: Scale Non-Linear function: input edge sensor analog, Output: Edge Position MM, Input Value:11471 13210 156694 21391 26289 32208 39272 45110 52250 59139 65160 65565, desired output: 0.00 0.06 1.48 4.74 7.55 10.94 14.98 18.33 22.42 26.36 29.82 30.00.

Rung 13: Scale Linear function: input: polypipe.ax2encpostion, output: Cutting Motor Degreesm, In min: 0, In Max: 400, Out Min: 360, Out Max: 0.

Rung 14: (NO) function named “Die Position Zero, Set Position Function: Module name: AMC-1, Channel/Axis: 3, Channel/Axis Name: Table Motor, set position to: 0.

Rung 15: (NO) function named “HomePosition” on column 1, Column 11: Find home (HOME) function. Find home: use structure = Edge_Home_Position, Module Name = AMC-1, Channel/Axis = 3, Channel/Axis Name = Table Motor, Mode = 2-Move to Switch 1 and Halt, Init Direction = Negative, Switch 1 Input = In3 – Home, Edge = Rising, First Speed = Speed1, Accel Rate = Accel, Set Position To = Position, In Progress = InProgress, Complete = Complete, Move Status = Status.

Rung 16: Math Function: Result: lowLimit, Formula: Die Index Target – 0.5.

Rung 17: Math Function: Result: HighLimit, Fomula: Die Index Target + 0.5.

Rung 18: Limit Value Function: input: Polypipe.Ax3Position, output: DieOutOfPosition, High Limit: HighLimit, Low Limit: LowLimit

Rung 19: Compare Contact (CMP) with the compare function being Polypipe.Ax3Position > HighLimit at column 1, Line ends with Debounce Coil (DBN) function, with the setup: (MoveDiePosition: On Time = 200, Off Time = 0)

Rung 19.1: line down from rung 18 at column 1,  Compare Contact (CMP) Polypipe.Ax3Position < LowLimit at column 2, line connects back to rung 18 at column 2.

Rung 20: Normally Open Edge Contact (NOE) function set on rising edge with the name MoveDiePosition. Out Coil (OUT) function used at the end with name CycleDie.

Rung 21: (NO) function named “Die Position Move Enable”, (NC) function named "CycleDie”, Simple Move Function with the setup: Module Name: Amc-1, Channel/Axis: 3, Channel/Axis name: Table Motor, Move Addressing: Absolute, Direction: N/A, target: Die Index Target, Velocity: 200, Acceleration: 100, Deceleration: 100, Stop Setup: Immediate Stop.

Rung 22: (NO) function named “Always Off Bit”, Simple Move Function with the setup: Module: amc-1, channel/axis: 3, channel/axis name: Table Motor, complete: target die position complete, move addressing: relative, direction: use target sign, target: Target Die Position Test, Velocity 0.1, Acceleration: 0.2, Deceleration: 0.2, Stop Setup Stop at Maximum Decel Rate.

Rung 23: (NO) named “Target Die Position Complete”. Reset Coil (RST) function named “Target Die Position Complete”

Rung 24: Math function with setup: Result: Target Die Position MM, Formula: Desired Die Position MM – (Edge Position MM + PolyPipe.Ax3Position).

Rung 25: Copy Data function with the setup = Source: Target Die Position MM, Destination: Die Position.Target, Type: Copy Value.

Rung 26: Normally Open Edge Contact (NOE) function with rising edge selected, named “PolyPipePositionSet”. Copy Data function with the setup = Source: PolyPipe.Ax4EncPosition and PolyPipe.Ax2EncPosition, Destination: PolyPipePositionCapture and Die Position Capture, type: Copy Value.

Rung 27: Math Function with the setup = Result: PolyPipePunchTrigger, Formula: PolyPipePositionCapture + ((Row Spacing Inches / 6.31974025)*360)

Rung 28: Campare Values function with the setup = Input 1: PolyPipe.Ax4EncPosition, Input 2: PolyPipePunchTrigger, Equal To: PolyPipePositionSet.

Rung 29: Math Function with the setup = Result: DieAccelDecelRate, Formula: PolyPipe.Ax4Velocity^2

Rung 30: Math function with the setup = Result: Die Position Target, Formula: Die Position Capture + 400.

Rung 31: (CMP) function: PolyPipe.Ax2Position ≥ Row Spacing Inches, (OUT) function named “Die Run”.

Rung 32: (NO) function named “Die Run”, (OUT) function named "Cutting Motor Off”.

Rung 33: Math function with the setup = Result: Die Index Target, Formula: (6.63 + Hole Size) + (Edge Position MM + PolyPipe.Ax3Position).

Rung 34: (NOE) function with rising edge selected, named “PolyPipe.Ax2In3”. Copy Data function: Source: Edge Position MM, Destination: Edge Position Reference Zero, Type: Copy Value.

Rung 35: Math Function: Result: Die Index Velocity, Formula: abs(Die Table Position MM – Die Index Target).

Rung 36: Math function = Result: Die Index Ramp Rate, Formula: Die Index Velocity^2.

Rung 37: Scale Linear function: Input: Die Table Position Analog, Output: Die Table Position MM, In Min: 13200, In Max: 55100, Out Min: 10.31875, Out Max: 0

Rung 38: CMP: TriggerPosition > Row Spacing Inches, Set Position: Module Name =AMC-1, Channel/Axis = 4, Channel/Axis Name = Encoder, Set Position To = 0.

Rung 39: end

Encoder has 2500 pules per revolution, it is on a 4in diameter roller, which should rotate approximately 3 rotations to make 38inches. If we have a die of 7 in diameter roller with a cutting die on it, and the speed of the 7in rollar is matching the speed of the 4in roller at all times. In the program we have this “Software:

Rung 1: a normally open contact (NO) that says "Top Motor On"  at the end of the line i have the word "enable" then a Velocity Move function named 'top motor' looking at the velocity move setup it shows axis name: Top Motor which is this device “Stepper motor 2” it is using a ‘Use Structure’ and the name is Top Motor. Move setup: direction = negative. Stop setup: stop at maximum decel rate.

Rung 2: normally open contact (NO) that is titled “Cutting Motor On”. At the end of the line there is a simple move function. On the simple move function we see: module name: amc-1, channel/axis: 1, channel axis name: Die Motor, move addressing: absolute, direction: negative, target: 1, velocity: PolyPipe.Ax4Velocity, acceleration: DieAccelDecelRate, Deceleration: DieAccelDecelRate, Stop Setup: immediate stop.

Rung 3: normally open contact (NO) that is titled “Table Motor On”. At the end of the line there is a simple move function. On the simple move function we see: module name: amc-1, channel/axis: 3, channel axis name: amc-1-axis-3, move addressing: relative, direction: use target sign, target: Die Position.target, velocity: 2, acceleration: 4, Deceleration: 4, Stop Setup: immediate stop.

Rung 4: normally open contact (NO) that is titled “TestHomeSwitch”. At the end of the line there is a set position function. In that function we have as follows: Module Name: AMC-1, Channel/axis:1, channel/axis name: Top Motor, set position to: 0,

Rung 5: on column 3 = NO Edge Contact (NOE) named PolyPipe.Ax1In3 with Rising Edge (on when bit turns on) selected, column 11 = Set Coil named StopCutCycle

Rung 5.1: drops down from rung 5 at column 4, on column 11 = Reset Coil named TriggerCut

Rung 5.2: drops down from rung 5 at column 4, on column 11 = Set Position (Axis/Channel Name: Encoder, Module Name: AMC-1, Channel/Axis: 4, Set Position to: 0)

Rung 6: normally open contact that is titled “TriggerCut” at the end of the line we have a reset coil function (RST) named StopCutCycle.

Rung 7: Math function. With the following setup. Result: TriggerPosition, formula: polypipe.ax4encposition / 12.56637.

Rung 8: Compare contact (CMP) function. With “triggerposition” on the left side of the function and “row spacing inches” on the right side of the function. The line ends with a Set Coil (SET) function named “TriggerCut”

Rung 9: (NO) function named “TriggerCut”, a Normally Closed Contact (NC) named “StopCutCycle”. And ends with a GEAR drivetrain function. The gear drivetrain is set up as such: Module Name: AMC-1, Drivetrain Segments: 1, Encoder as Primary Master Axis: Yes, Primary = (Master: Amc-1axis-4, Slave AMC-1axis-1, Ratio 40.3299, disable)

“ with Row Spacing Inches set to 38, why does the 7 inch roller with die take more than 38 counts to make a complete rotation?

-40.3299

40.3299

Hardware:

- productivity2000

- P2-550 CPU

  - P2-04AD

    - Location: 4 slot base board

    - Task: gathers data from Roller Encoder

  - P2-08TRS

  - P2-08SIM

- Strider (SE2-SW5UG-T)

- Amc4 connected with

  - Encoder

  - Stepper motor 1 + Encoder

    - Location: bottom of the device

    - Task: rotate the cutter die at the same velocity of material moving across the platform (rotate only when cut needed)

    - Connected to: AMC-1-axis-1

  - Stepper motor 2

    - Location: top of the device

    - Task: match the velocity of the polypipe (constant rotation)

    - Connected to: AMC-1-Axis-2

  - Stepper motor + Encoder

    - Location: side of platform

    - Task: move platform horizontally to material

    - Connected to: AMC-1-Axis-3

    - SureMotion linear actuator assembly

- Roller Encoder

  - Location: connected to roller that material turns

  - Task: gather velocity of material

- Connecting AutomationDirect photoelectric fork sensor: (+) wire to P2-04AD 24V+ port. Signal wire to AMC-1-Axis-1 port 7 (IN3 Home), known in program as PolyPipe.Ax1In3

- Gefran potentiometer linear position sensor

Here is what we have. We have a Stepper motor connected to a SureMotion linear actuator assembly. The Stepper Motor has a encoder connected to it. This encoder should tell us the position of the SureMotion linear actuator assembly. The Linear actuator is connected to a table that has a implement on it which is known as ‘Cutting Die’. The cutting die is a 1in in diameter metal circle. There is a Gefran potentiometer linear position sensor also connected to the table. This Gefran potentiometer linear position sensor should tell us the location of the table. There is a material called ‘Polypipe’ which move over the table. There is a Sick Array sensor: AS30-EBM314I220A00 which reads where the edge is in relationship to the sensors width readings. We need a program to do is read the positions of these three components to set the edge of the cutting die always equal to the edge of the material, unless we specify how much we want the cutting die to index into the material. For example we want the die to move so that 1/4th of the die is under the material. This “1/4th” number will be desinated in inches and will be selected in the program. The die needs to move in real time. since the material is always moving across the table, the material will not always stay at the same position that it started on in the Sick Array Sensor. Therefore the table needs to constantly move to make the edge or the desired value be always in the correct position.

We have a system setup consisting of several components:

1. **Stepper Motor and SureMotion Linear Actuator Assembly**:

- The system includes a stepper motor that drives a SureMotion linear actuator.

- This stepper motor is equipped with an encoder that provides positional feedback, allowing us to determine the position of the linear actuator assembly.

2. **Cutting Die and Table Assembly**:

- The linear actuator moves a table that holds a component known as the "Cutting Die."

- The Cutting Die is a metal circle with a diameter of 1 inch.

- A Gefran potentiometer linear position sensor is also attached to the table to give us precise feedback on the table's position.

3. **Material Movement and Edge Detection**:

- The material, called "Polypipe," moves over the table.

- A Sick Array sensor (model AS30-EBM314I220A00) is used to detect the position of the material’s edge relative to the sensor’s width readings.

### Objective:

We need to develop a program that reads the positions from three components: the stepper motor's encoder, the Gefran potentiometer, and the Sick Array sensor. The goal is to align the edge of the Cutting Die with the edge of the moving material. If a specific offset is required, the program should allow the die to index into the material by a user-specified distance.

For example, if we want 1/4 of the Cutting Die to be under the material, this offset (specified in inches) should be adjustable in the program. The system must continuously adjust the die’s position in real-time to maintain the desired alignment as the material moves across the table.

### Key Points:

- **Real-Time Adjustment**: The die needs to move in real-time to compensate for the material's movement across the table.

- **Edge Alignment**: The edge of the die should match the edge of the material, unless an offset is specified.

- **Dynamic Tracking**: Since the material's position relative to the Sick Array Sensor may change, the table must adjust continuously to keep the die in the correct position.

---

I have a

Table Stepper Motor + Encoder connected: Amc-1-Axis-3.

Sick Array sensor (model AS30-EBM314I220A00) connected: P2-04AD port 3 & 12

Gefran potentiometer linear position sensor connected: FC-P3  P2-04AD port 5&10.

Here is the way it is set up. When the values for the potentiometer increase, the array sensor (edge sensor) decreases. Say I want to write an equation or have a relationship between the two sensors so that they move together. Say if the edge sensor moves from 20mm to 19mm, and 20mm of the edge sensor is 0 mm for potentiometer, then it is reasonable to say that if we go from 20mm  19mm, we should go from 0mm  1mm with the potentiometer. Correct? How would I write this relationship

so the linear potentiometer moves from 0 through 1000 without reseting to 0 along the way. however since the stepper motor is rotational it moves from 0 to 360 before reseting to 0, this cycle continues until the linear actuator moves from 0mm to 30mm. with this type of understanding how do we do this " I don't really know what i am trying to figure out please help. I have automation direct stepper motor and potentiometer, both give me a value in the ladder logic of where they are in pulses. Since the potentiometers values are more refined i want to use its position numbers for more precise measurements. However, i need to figure out how to make the number from teh potentiometer control the stepper motor, so that it moves my device to the given location.”