Measuring Protocol for Hole Size and Spacing Analysis

### Purpose:

To accurately measure hole diameters and spacing between holes on a sample pipe or sheet using a standard ruler (in 1/64 inch increments) and input values into a preformatted Excel sheet.

### Tools Required:

- 1/64 inch increment ruler (or caliper with 1/64 in resolution)

- Calculator (optional, for conversions)

- Preformatted Excel file with the following columns:
row, value, hole_d, hole_r, s_v_in, s_v_sub_in, spacing

### Measurement and Entry Instructions:

#### 1. Measuring Hole Diameter

- Place the ruler across the widest point of the hole (the diameter).

- Read the value in 1/64 inch increments.

- Record this integer value in the value column.

  - Example: If the hole measures 50/64 in, input 50.

- Excel will calculate:

  - hole_d = value / 64

  - hole_r = hole_d / 2

#### 2. Measuring Spacing Between Holes

- Place the tape measure between the Far edge of one hole and the near edge of the next hole.

- Record the whole number of inches in the s_v_in column.

  - Example: If the spacing is 25 4/32 inches, input 25 in s_v_in.

- Record the numerator of the fractional inch in the s_v_sub_in column (base 32).

  - Example: For 4/32 inches, input 4.

#### 3. Repeat Measurements

- Repeat steps 1–2 for each row (i.e., each hole and its spacing to the next one).

- Ensure consistent measuring direction (e.g., left to right or top to bottom).

- Continue until all rows in the Excel file are filled or the sample is complete.

### Important Notes:

- Measure hole-to-hole from edge to edge, not center to center.

- The formula corrects your edge-to-edge measurement into center-to-center spacing by adjusting for each hole's radius.

- Be consistent with edge selection (e.g., always measure from the right edge of one hole to the left edge of the next).

### Quality Control:

- After entering data, review Excel-calculated spacing column to ensure values are reasonable (~25 in).

- Check if the average, desired final, and actual final values update correctly.

- If any spacing values seem off (e.g., <24 or >26), re-measure those entries for confirmation.